RT-PCR PROTOCOL  in  Dictyostelium cells

Sergio Carilla

- Mix 1 g RNA (0.5 l of 2g/ml), 12 l H2O DEPC and 1l of reverse specific primer

- Heat at 70ºC 5 min in thermocycler and cold on ice for 5 min

- In the same tube add the following mix to final volume of 25 l

5X buffer              5l

RNAse Out           1l

dNTPS (10 mM)  2.5l

AMV                     1l

DEPC H2O           2 l

- In a thermocycler use the following programme:

30 min 37ºC

1.30 h  42ºC

5 min   80 ºC

5 min     4ºC

- Use 2 ml of cDNA from step 4 to the next mix:

cDNA          2					95 º  5min

dNTPS        5					95º   1min

buffer          5					45º    1min

oligos          1+1				62º    X min

Taq             1					62º    10 min

H2O             35				4º       ∞