SITE DIRECTED MUTAGENESIS

1. 	Design two complementary oligonucleotides according the following instructions:

- There must contain the mutation in the middle of the sequences

- It should have between 25 to 40 bp

- Must have at least 40% G+C and finish in one or more G or C.

- Tm with 10ºC over the extension temperature (62ºC)

2.	Prepare the following mix:

| Components | l |
| --- | --- |
| Buffer 5X HiFi FID | 10 |
| plasmid template mini diluted 1/50 | 2 |
| Oligo MF 50M | 2,5 |
| Oligo MR 50M | 2,5 |
| dNTPS 10mM | 3 |
| Pfu HiFi 1U/l (KAPA Biosystems) | 2,5 |
| H2O | 27,5 |
| Final volume | 50 |

3.	Run the following program:

95ºC 	5min

95ºC	20s

X 18

45ºC	1min

62ºC	30sec/Kb

62ºC 	5min

4ºC	∞

4.	Digest 2 hours with 10 U DpnI (Invitrogen ; 5U/l) in the same PCR tube.

5.	Purify DNA with a Quiagen column and elute in 30 l buffer EB (Quiagen).

6.	Transfect 25 l from step 5 in DH5 E.coli strain.

7. 	Grow in LBA plates O/N (see the antibiotic resistance in your particular case).

8. 	Select colonies (the plate should be full of colonies) , grow in LB O/N and

extract the plasmid DNA by miniprep.

9. 	Sequence the DNA to confirm the mutation.

NOTE 1: Altough the Kapa HiFi polymerase has high fidelity is better to use some small plasmid like P-GEMT-easy (Promega; aprox 3Kb). The best is to have the wild type sequence in this vector, perform the mutagenesis and then cut and ligate it (both wild type and mutated forms) in a Ptx-GFP- like vector or in a PDV-GFP-CTAP vector to transfect it in Dictyostelium.

NOTE 2: It´s not necessary to use IPTG or XGAL in the case you use P-GEMT-easy as a template.